# IMPORTANT NOTES FOR Updating and Downgrading MC9590/9596/9598 Device to/from BSP41 (v4.41.03) RevD Software.

Upgrading

- If your device is running BSP27 (v2.27.08) / BSP37(v3.37.02) Windows Mobile 6.5, you can simply use the BSP41 (RevD SW v4.41.03) Update Loader package to update from BSP27/BSP37 to BSP41

- If your device is running BSP19 (v1.19.11), you must first update your device to the released version of BSP27 (v2.27.08).  Once the device is running BSP27 (v2.27.08), you can then perform the update to BSP41 (v4.41.03).

Downgrading

- If your device was manufactured before Dec1,2012 and your device has 256MB RAM/LG Display, you are only permitted to downgrade to BSP27 (v2.27.08) / BSP37(v3.37.02)

(Go to System Info Applet on the device to find the RAM size: Start->Settings->System->”System Info” Applet->”Misc” Page)

(Go to System Info Applet on the device to find the Display Id: Start->Settings->System->”System Info” Applet->”Display”.  For CMI-> PJ037PD, For LG-> LH370V01)

- If your device was manufactured on or after Dec 1,2012 and your device has CMI display, you WOULD NOT be able to  Downgrade to BSP37(v3.37.02) / BSP27 (v2.27.08) / BSP19 (1.19.11)

(DO NOT Downgrade MC95xx devices built on or after Dec 2012 with the new CMI display to BSP37 (v3.37.02), BSP27 (v2.27.08), BSP19 (1.19.11) since these BSPs do not have support for CMI display)

(Go to System Info Applet on the device to find the Display Id: Start->Settings->System->”System Info” Applet->”Display”  For CMI-> PJ037PD, For LG-> LH370V01)

- If your device has 128MB RAM and running with BSP41 (v4.41.03), you are permitted to downgrade to BSP37 (v3.37.02), BSP27 (v2.27.08), BSP19 (1.19.11).  However, if you want to downgrade to BSP19 (1.19.11), you must first downgrade to BSP27 (v2.27.08) before performing the downgrade to BSP19 (v1.19.11).

(Go to System Info Applet on the device to find the RAM size: Start->Settings->System->”System Info” Applet->”Misc” Page)

Preserving Application partition:

- CleanAppl.bin must be removed from Pkgs.lst in the Update Loader Package to preserve Application partition data.